# ComStock Change

v32 – OpenStudio Standards 0.7.0 Update.

Implementer: Matthew Dahlhausen

Change Document Author: Matthew Dahlhausen

Reviewer: Chris CaraDonna, Eric Ringold

# Description of Change

## Summary

This updates ComStock to the latest openstudio-standards release (v0.7.0). There are few changes from 0.6.3, namely some bug fixes when equipment sizes were too large for the lookups in some building types.

## Existing Methodology

ComStock currently uses OpenStudio v3.8 with an older version of openstudio-standards, v0.6.3.

## New Methodology

This measure updates ComStock to the latest version, openstudio-standards (v0.7.0). There were no major changes to workflow or measures, only bug fixes.

# Impact of Change

## Individual Model Input Change

## Stock-Level Input Change

Not applicable; no changes to sampling.

## Model Changes

These PRs impact ComStock models:

- #1804 fixed PTHP capacity and VAV minimum damper calculation issues

- #1805 fixed refrigerated case and walkin defrost and dripdown schedule values

- #1834 fixed a UA calculation error in FFactor constructions

- #1838 default WSHP efficiency if capacity too large

- #1839 add lookup for larger DEER motors

All of these are bug fixes. The last three deal directly with issues that caused simulation failures in ComStock runs.

# Stock-Level Output Change

The comparison plots show (4) separate run conditions:

- CBECS 2018 baseline

- Current ComStock baseline used in the most recent EUSS runs, using OpenStudio v3.8.0 and openstudio-standards v0.6.3

- ComStock main as of Nov/Dec 2024, using OpenStudio v3.8.0 and updating to openstudio-standards v0.7.0

- ComStock main as of Nov/Dec 2024, updating to OpenStudio v3.9.0 and openstudio-standards v0.7.0

The failure rates are shown in the following table:

| run | Building Count | Success | Failed | Fraction of Total Failed |
| --- | --- | --- | --- | --- |
| os_380_com_4d7baa7_stds_063 | 11078 | 10699 | 379 | 0.034 |
| os_380_com_01cc36b_stds_070 | 11078 | 10731 | 347 | 0.031 |
| os_390_com_32b3f6f_stds_070 | 11078 | 10433 | 645 | 0.058 |

There are few differences in failure rates. There was an uptick in models failing in OpenStudio v3.9.0 because of a pump heat buildup simulation error in models with ‘DOAS with water source heat pumps cooling tower with boiler’ HVAC systems. All water source heat pumps systems are affected.

Total fuel uses – electricity and natural gas – are identical are extremely similar across building types, census regions, and vintages.  There are slight changes in the OS v3.9.0 update with buildings that are more likely to use WSHPS (large hotels) because of the higher failure rate.

There remain large errors in the Pacific region, warehouses, full-service restaurants, hospitals, and schools.

Close inspection of end use intensities shows most of the change from fan and refrigeration energy use. In particular, refrigeration energy use is now present in strip malls from the restaurant component.

| OS version | 3.8.0 | 3.8.0 | 3.8.0 | 3.8.0 | 3.8.0 | 3.8.0 |
| --- | --- | --- | --- | --- | --- | --- |
| os stds version | 0.7.0 | 0.8.2 | 0.7.0 | 0.8.2 | 0.7.0 | 0.8.2 |
| end use | fan | fan | refrigeration | refrigeration | ext lighting | ext lighting |
| unit | kwh_per_ft2 | kwh_per_ft2 | kwh_per_ft2 | kwh_per_ft2 | kwh_per_ft2 | kwh_per_ft2 |
| building_type |  |  |  |  |  |  |
| strip_mall | 4.639 | 5.956 | 0.000 | 1.276 | 1.571 | 1.440 |
| retail | 3.711 | 4.754 | 0.000 | 0.000 | 1.438 | 0.839 |
| warehouse | 0.550 | 0.860 | 0.000 | 0.000 | 1.160 | 0.604 |
| small_office | 2.844 | 4.570 | 0.000 | 0.000 | 1.061 | 0.767 |
| full_service_restaurant | 11.775 | 12.240 | 2.258 | 2.618 | 1.443 | 1.208 |
| quick_service_restaurant | 12.990 | 15.471 | 5.323 | 5.931 | 2.535 | 2.179 |
| grocery | #DIV/0! | 6.213 | #DIV/0! | 18.486 | #DIV/0! | 0.643 |
| large_hotel | 2.688 | 1.622 | 0.641 | 0.710 | 0.637 | 0.459 |
| small_hotel | 1.932 | 1.336 | 0.000 | 0.000 | 1.681 | 1.229 |
| medium_office | 2.892 | 3.087 | 0.000 | 0.000 | 0.624 | 0.506 |
| primary_school | 4.132 | 4.209 | 2.122 | 1.274 | 1.157 | 0.664 |
| large_office | 2.549 | 1.999 | 0.000 | 0.000 | 0.368 | 0.310 |
| secondary_school | 3.691 | 4.326 | 1.815 | 0.963 | 1.021 | 0.622 |
| outpatient | 4.366 | 6.610 | 0.000 | 0.000 | 0.814 | 0.771 |
| hospital | 3.957 | 4.754 | 0.463 | 0.259 | 0.491 | 0.364 |

Gas end use intensities changed slightly, trending lower with water heating. This is from changes in the service water heating module.

| OS version | 3.8.0 | 3.8.0 | 3.8.0 | 3.8.0 |
| --- | --- | --- | --- | --- |
| os stds version | 0.7.0 | 0.8.2 | 0.7.0 | 0.8.2 |
| end use | gas_equip | gas_equip | water_heating | water_heating |
| unit | therm_per_ft2 | therm_per_ft2 | therm_per_ft2 | therm_per_ft2 |
| building_type |  |  |  |  |
| strip_mall | 0.094 | 0.101 | 0.132 | 0.095 |
| retail | 0.000 | 0.000 | 0.000 | 0.000 |
| warehouse | 0.000 | 0.000 | 0.000 | 0.000 |
| small_office | 0.000 | 0.000 | 0.006 | 0.004 |
| full_service_restaurant | 0.874 | 1.046 | 0.377 | 0.220 |
| quick_service_restaurant | 1.034 | 1.094 | 0.135 | 0.124 |
| grocery | #DIV/0! | 0.018 | #DIV/0! | 0.001 |
| large_hotel | 0.148 | 0.153 | 0.126 | 0.130 |
| small_hotel | 0.070 | 0.067 | 0.086 | 0.043 |
| medium_office | 0.000 | 0.000 | 0.006 | 0.007 |
| primary_school | 0.012 | 0.013 | 0.031 | 0.016 |
| large_office | 0.000 | 0.000 | 0.003 | 0.006 |
| secondary_school | 0.015 | 0.012 | 0.034 | 0.013 |
| outpatient | 0.034 | 0.035 | 0.010 | 0.011 |
| hospital | 0.158 | 0.217 | 0.071 | 0.038 |

# EUSS Upgrade Measure Impacts

There are no expected changes to EUSS measures.